Blood Flow Analysis Software Based On Duplex Doppler Ultrasound Recordings

Team 12

Stakeholder: Taha Abbasi Hashemi

Instructors: Dr. Colin McDonald, Dr. Spencer Smith

Group Members:

| Name: | MacID: | Program |
| --- | --- | --- |
| Seaya Liu | liu1150 | Software&BME |
| Kyle Hagerman | hagermak | Software&BME |
| Baoze Lin | linb44 | Software&BME |
| Ursula Chui | chuiu | Software&BME |

## Motivation (Problem or Needs Identification)

In vascular research and clinical diagnostics, analyzing blood flow and vascular morphology is critical for identifying cardiovascular pathologies. Currently, the extraction of hemodynamic metrics from duplex Doppler ultrasound recordings is a highly manual, time-intensive process. Vascular laboratories capture hundreds of video files containing both B-mode (brightness mode) structural imaging and Doppler velocity waveforms. Technologists and researchers are frequently forced to use disjointed tools to measure vessel diameter, estimate cross-sectional area (CSA), and extract blood velocity over time to calculate volumetric flow.

This fragmented workflow introduces human error and limits the throughput of data processing in research environments. Furthermore, while individual ultrasound machines capture simultaneous data streams, such as electrocardiogram (ECG) signals alongside B-mode and Doppler data, integrating and syncing these modalities for bulk post-processing remains a significant challenge. There is a need for a unified graphical software application capable of batch-processing ultrasound video files, applying automated feature extraction, and preserving a human in the loop interface for clinical validation.

## Solution Statement (Design Aspect of the Project)

To address these inefficiencies, our team will design and develop an end-to-end graphical user interface (GUI) and data processing pipeline using Python, integrated with the vascular AI lab’s existing custom software, PhysioMerge. The application will treat each video as an independent data source, allowing researchers to batch select directories of ultrasound recordings for automated processing.

### Core Processing Workflows

1. Longitudinal View Analysis: The software will process longitudinal recordings containing both B-mode structural data and Doppler waveforms.

- Spatial Calibration: The system will dynamically read the pixel to millimeter ratio from the ultrasound's on-screen ruler, mapping pixel length to physical distance.

- Diameter Extraction: By applying image processing techniques, the software will isolate the vascular walls to determine the perpendicular distance between them.

- Hemodynamic Calculation: The system will extract the time-averaged blood velocity from the Doppler waveform. By combining the calculated diameter and velocity, the software will output an estimate of volumetric blood flow.

2. Transverse View Analysis: Operating on B-mode only, the software will analyze the vessel as a circular structure. Edge detection algorithms will outline the inner lumen of the vessel to compute the precise cross-sectional area. The GUI will allow the user to define an initial region of interest (ROI) and subsequently adjust the automated edge detection spline, ensuring clinical accuracy.

### Advanced Signal Processing Extensions

Beyond the core scope, the architecture will be designed modularly to support advanced feature extraction:

- ECG Processing: Integration of the Pan-Tompkins algorithm to detect R-peaks from the on-screen ECG trace.

- Doppler Waveform Analysis: Automated extraction of the middle envelope of the Doppler signal to categorize flow as monophasic, biphasic, or triphasic, as well as calculating morphological data from the waveforms. This functionality will be designed to integrate seamlessly with PhysioMerge, the internal application developed by the lab for their existing analysis workflows, allowing the two tools to share data and expand the lab's capabilities.

- Vascular Profiling: Support for Intima-Media Thickness (IMT) calculations to measure the vascular wall thickness directly from B-mode data.

## Educational Background and Expected Contributions

As a team comprised exclusively of Software Engineering students, our discipline aligns perfectly with the architectural, algorithmic, and user experience demands of this project. While the domain involves physiological data, the core challenges involve state management, image and signal processing, automated batch pipelines, and interactive UI/UX design. These are foundational software engineering problems.

Our educational background has equipped us with formal training in software design patterns, algorithm optimization, and human-computer interaction (HCI), plus internship experience at large companies. We have experience in efficient Python integrations and scalable software architectures. This ensures the application remains responsive even when processing heavy computational workloads like continuous edge detection.